# PILOT CAR REQUIREMENTS

One (1) Pilot Car Required:

A.	Length:

- Any vehicle with a single kingpin combination between 75’ to 100’

- Any vehicle combination of truck, tractor, and jeep/semi-trailers coupled together (multi-kingpin) exceeding 120’

- Any vehicle combination of truck, tractor, jeep, and semi-trailers coupled together (multi-kingpin), with articulating rear steering on the rear semi-trailer between 125’ to 135’.

B.	Width:

- Loads between 12’ to 15’

C.	Height*

1.	Front pilot car with vertical measuring device on loads exceeding 17’

D.	Overhang (Hauled Loads):

- Rear – when the rear load projection exceeds 25’ when measured from the rear extremity of the hauling equipment;

- Front – when the front overhang exceeds 25’

Two (2) Pilot Cars Required:

A.	Length:

- Any vehicle with a single kingpin combination exceeding 100’

- Any vehicle combination of truck, tractor, jeep, and semi-trailer coupled together (multi-kingpin), with articulating rear steering on the rear semi-trailer exceeding 135’.

B.	Width:

- Loads exceeding 15’

C.	Combination* - A combination of the following:

- 1C (Height) and 1A (Length), or

- 1C (Height) and 1B (Width)

Notes:		* - 1C or 2C movement is restricted to midnight to 6:00 a.m.